# English Edited Dialogue Record
Doxycycline Hyclate Tablet Disintegration Analysis

Scope: This document includes the English-edited content of the drug-disintegration discussion only. The final Chinese request to generate this Word document is excluded.

# 1. Task and Analytical Framework

User

I need you to analyze the disintegration images of two groups of Doxycycline Hyclate Tablets, Group A and Group B. These may be images of the same drug or of different drugs. I will upload five images for each group, with each picture taken 15 minutes apart. Please describe the drug disintegration process in English for each image in each group, and finally determine whether the two sets of images are from the same drug.

You may call the Knowledge Base to remember how to describe drug disintegration. As you describe the tablets in English, pay attention to changes in the size and shape of the tablets. If you are prepared, I will upload Group A, Picture 1 at 0 minutes.

Assistant

I am prepared. Please upload Group A, Picture 1 at 0 minutes.

I will describe each image in English using the eight disintegration dimensions: color change, shape change, surface texture change, volume/size change, dissolution speed and time, physical state change, dissolution medium, and fragment distribution/density. I will pay special attention to tablet size, outline, cracking, swelling, erosion, fragmentation, and residue distribution.

After all ten images are uploaded, A1 to A5 and B1 to B5, I will compare the two time series and judge whether they are likely from the same drug formulation or from different drugs/formulations based on visible disintegration behavior rather than chemical confirmation.

User

You are not fully prepared. Please call the Knowledge Base again to remember how to describe drug disintegration.

Assistant

I have refreshed the full drug-disintegration description framework from the Knowledge Base.

I will analyze every uploaded image using the eight required dimensions: Color Change, Shape Change, Surface Texture Change, Volume Change, Dissolution Speed and Time, Physical State Change, Dissolution Medium, and Fragment Distribution with Density. The guide recommends using these dimensions systematically to capture the full disintegration process, not only one visible feature.

For each image, I will describe:

Color change: tablet color, liquid transparency or turbidity, and local or spreading color changes.

Shape change: whether the tablet remains intact, becomes distorted, cracks, erodes, or fragments.

Surface texture change: smoothness, roughness, pores, peeling, cracks, and powdery or flaky surfaces.

Volume and size change: reduction in tablet diameter, thickness, height, edge sharpness, swelling, or collapse.

Dissolution speed and time: because each picture is 15 minutes apart, I will compare visible breakdown across 0, 15, 30, 45, and 60 minutes.

Physical state change: swelling, softening, cracking, internal weakening, collapse, and powder formation.

Dissolution medium: clarity, turbidity, suspended matter, sediment, floating material, and visible interaction between tablet and liquid.

Fragment distribution and density: where fragments appear, whether they remain near the tablet, spread outward, settle at the bottom, or form cloudy suspension.

For the final comparison between Group A and Group B, I will judge whether they are likely the same drug/formulation or different by comparing the full time-sequence pattern: initial tablet size and shape, erosion speed, swelling behavior, cracking pattern, fragment density, remaining core size, sediment/cloudiness, and final residue distribution. Size and shape are especially important because they strongly affect disintegration behavior and can support visual identification.

I am ready. Please upload Group A, Picture 1 at 0 minutes.

# 2. Group A Images and Description

User

Here are the Group A pictures. When describing the drug, please make sure the English description is accurate. Think carefully and reduce hallucinations. Each picture was taken 15 minutes apart.

## Uploaded Group A Image Sequence

| Group A - 0 minutes | Group A - 15 minutes |
| --- | --- |
| Group A - 30 minutes | Group A - 45 minutes |
| Group A - 60 minutes |  |

Assistant

Below is my visual analysis of Group A only. I will avoid claiming chemical identity from the images alone. The description follows the eight-dimension disintegration framework: color, shape, surface texture, volume/size, dissolution speed/time, physical state, medium, and fragment distribution.

## A1 - 0 minutes

Color change: The tablet appears uniformly white. No obvious yellow core or colored diffusion is visible yet. The surrounding medium appears mostly clear against the dark background.

Shape change: The tablet is still intact, with a thick, biconvex body and a relatively well-defined side wall. The outline is smooth and continuous, with no visible breakage.

Surface texture change: The surface looks mostly smooth and compact, suggesting an intact coating or compressed outer layer. Only very slight surface unevenness is visible under the lighting.

Volume / size change: This is the baseline size. The tablet retains its full height and width, with no visible swelling, collapse, or erosion.

Dissolution speed and time: At 0 minutes, there is no clear disintegration yet. The process appears to be at the pre-penetration or very early wetting stage.

Physical state change: The tablet remains solid and structurally stable. No cracking, softening, powdering, or core exposure is visible.

Dissolution medium: A few small bubbles are visible around the tablet surface, but the medium itself appears clear. No obvious cloudiness is seen.

Fragment distribution and density: No detached fragments are visible. Particle density around the tablet is minimal.

## A2 - 15 minutes

Color change: A major color change has appeared. The original white exterior is partly lost, and pale yellow internal material is exposed over much of the upper surface. The tablet now has a white-to-yellow mottled appearance.

Shape change: The original smooth tablet shape has changed into a rounded, swollen, irregular mound. The sharp side wall and smooth biconvex outline are no longer preserved.

Surface texture change: The surface becomes highly rough, porous, and fluffy/granular. The outer layer appears loosened, with powdery material covering the surface.

Volume / size change: The tablet appears to have expanded or swollen outward while also losing its original compact outline. Its height remains substantial, but the structure is less dense and more sponge-like.

Dissolution speed and time: Between 0 and 15 minutes, the disintegration rate is clearly rapid. Liquid penetration likely caused swelling, coating disruption, and exposure of the yellow interior.

Physical state change: The tablet changes from a compact solid into a softened, porous, partially collapsed mass. The yellow core-like region is now visible, indicating that the internal structure has been exposed.

Dissolution medium: The medium contains visible fine suspended particles. Below the tablet, faint downward trails suggest material dispersing into the liquid.

Fragment distribution and density: Fragments and powder are concentrated near the tablet surface and below the support line. Several larger detached white fragments are visible on the lower right, while finer particles are suspended around the tablet.

## A3 - 30 minutes

Color change: The tablet remains mainly pale yellow with residual white areas, especially near the lower and side regions. The yellow coloration is more evenly visible than at 15 minutes.

Shape change: The mass is still mound-like but appears less sharply defined. The original tablet geometry is completely lost. The top is rounded, but the edges are uneven and eroded.

Surface texture change: The surface remains coarse, porous, and powdery. Many small granules are visible on the surface, giving it a sponge-like or crumbly texture.

Volume / size change: Compared with 15 minutes, the visible mound appears slightly lower and more eroded/spread. The tablet has not fully dissolved; a large residual body remains.

Dissolution speed and time: The disintegration continues but appears slower than the initial 0-15 minute interval. The fastest visible structural breakdown likely occurred before 15 minutes; by 30 minutes, erosion and particle shedding dominate.

Physical state change: The tablet is now a semi-cohesive wet granular mass rather than a hard tablet. The structure still holds together, but its surface is weakened and continuously shedding fine particles.

Dissolution medium: The medium shows many suspended specks. Downward streaks beneath the tablet suggest continuing release of fine particles or dissolved material.

Fragment distribution and density: Fragments are densest immediately around and beneath the tablet. Fine particles are scattered throughout the field, while heavier material appears to settle below the support line.

## A4 - 45 minutes

Color change: The residue is mostly light yellow to off-white, with yellow becoming more diffuse across the remaining mass. There is no strong darkening; the color remains pale.

Shape change: The remaining tablet mass is a low, rounded, irregular mound. A noticeable eroded depression or concave region appears on the left side, indicating localized collapse or material loss.

Surface texture change: The surface remains rough and granular. The top looks less compact, with many tiny particles attached to the surface.

Volume / size change: The visible volume appears reduced compared with the original intact tablet, but substantial residue remains. The height is lower than the original tablet, and the outline is flattened along the bottom.

Dissolution speed and time: From 30 to 45 minutes, disintegration appears gradual rather than dramatic. The tablet continues to erode and shed material, but the central mass persists.

Physical state change: The tablet has become a fragile, softened aggregate. It is no longer a solid tablet but has not fully dispersed into the medium.

Dissolution medium: The medium contains fine suspended particles. The area below the tablet shows sediment-like accumulation and faint vertical dispersion trails.

Fragment distribution and density: Particle density remains highest near the lower edge of the tablet and below the support. Finer particles are dispersed more widely in the medium, while larger residues remain close to the original tablet position.

## A5 - 60 minutes

Color change: The remaining mass is still pale yellow/off-white. Yellow coloration is visible throughout the central region, while some white granular material remains on the surface and edges.

Shape change: The mass remains as an irregular dome-like residue, not fully dissolved. The top surface has uneven protrusions and small raised granular peaks, especially near the upper left and right regions.

Surface texture change: The surface is very rough, fluffy, and particulate. It looks like a loose, porous matrix rather than a compact tablet.

Volume / size change: The tablet has lost its original shape and compactness, but the remaining residue is still sizeable. Compared with 45 minutes, the overall footprint appears similar, with only modest further reduction.

Dissolution speed and time: By 60 minutes, the process appears to have entered a slower late stage. The initial rapid breakdown has already occurred, and the remaining material dissolves or disperses more slowly.

Physical state change: The tablet is now a soft, partially disintegrated residue with persistent cohesive mass. It has not completely dispersed after 60 minutes.

Dissolution medium: The medium remains filled with scattered fine particles. The bottom region contains residue and sediment-like material, especially beneath the tablet.

Fragment distribution and density: Fragments are still concentrated near the tablet and below it. Fine particles are suspended throughout the surrounding medium, but the highest density remains around the original tablet location.

## Overall Group A Disintegration Pattern

Group A shows a rapid early transformation between 0 and 15 minutes: the intact white tablet loses its smooth outer structure, exposes a pale yellow interior, swells, and becomes a rough granular mound. From 15 to 60 minutes, the process becomes slower. The tablet continues to erode, shed fine particles, and release sediment downward, but a large cohesive residue remains even at 60 minutes.

The most important visual features of Group A are:

Initial tablet: white, smooth, intact, and thick with a biconvex form.

After 15 minutes: rapid swelling/softening and yellow core exposure.

Later stages: persistent pale yellow granular mound, slow erosion, fine suspended particles, and sediment below.

At 60 minutes: not fully dissolved, with a sizeable porous residue still present.

# 3. Group B Images and Description

User

Here are the Group B pictures. When describing the drug, please make sure the English description is accurate. Think carefully and reduce hallucinations. Each picture was taken 15 minutes apart.

## Uploaded Group B Image Sequence

| Group B - 0 minutes | Group B - 15 minutes |
| --- | --- |
| Group B - 30 minutes | Group B - 45 minutes |
| Group B - 60 minutes |  |

Assistant

I will treat the uploaded sequence as Group B at 0, 15, 30, 45, and 60 minutes, and then compare it with Group A. The description follows the eight-dimension drug-disintegration framework: color, shape, surface texture, volume, dissolution speed/time, physical state, dissolution medium, and fragment distribution/density.

## B1 - 0 minutes

Color change: The tablet is already pale yellow at the starting point. This is different from Group A, whose initial tablet appeared white. The medium around it looks mostly clear.

Shape change: The tablet is flat, wide, and cylindrical, with a relatively straight vertical side wall and a slightly rounded upper surface. It is not biconvex in the same way as Group A.

Surface texture change: The surface appears compact but not glossy. It has a slightly matte, fine-granular texture. No obvious cracks or detached fragments are visible yet.

Volume / size change: This is the baseline for Group B. The tablet appears wider and flatter than Group A's initial tablet.

Dissolution speed and time: At 0 minutes, there is no major visible disintegration, but the surface already looks wetted.

Physical state change: The tablet remains structurally intact. No swelling, collapse, or exposed fragmented region is visible.

Dissolution medium: The medium appears clear, with only small bubbles and minor floating specks.

Fragment distribution and density: Detached fragments are minimal or absent. Particle density around the tablet is very low.

## B2 - 15 minutes

Color change: The tablet shows a white outer swollen layer with a yellow internal band or core region visible through the middle. The color distribution is layered: white on the exterior and yellow in the central area.

Shape change: The original flat cylinder has expanded into a thicker, softened rectangular mound. The top and side edges are no longer sharp; the outline is blurred by swelling and surface erosion.

Surface texture change: The surface becomes fluffy, porous, and powdery, especially along the top and side edges. The surface looks less compact than at 0 minutes.

Volume / size change: The tablet appears swollen and slightly enlarged in height, likely due to water absorption and loosening rather than true structural preservation.

Dissolution speed and time: From 0 to 15 minutes, Group B undergoes rapid wetting and swelling. The outer layer begins to disintegrate strongly.

Physical state change: The tablet has changed from a compact solid into a soft, hydrated, porous mass. The yellow inner material is partly exposed but still surrounded by white material.

Dissolution medium: Many fine particles are visible in the surrounding medium. The medium is no longer visually clean; it contains suspended material.

Fragment distribution and density: Fine particles are dispersed above and around the tablet. Most solid material still remains close to the original tablet position.

## B3 - 30 minutes

Color change: The central residue remains yellow, while the surrounding broken material is mostly white. The yellow area is still visible but reduced and less sharply defined.

Shape change: The tablet has lost most of its original cylindrical shape. It is now a low, irregular mound, with detached piles on both sides.

Surface texture change: The surface is highly granular, loose, and eroded. The fluffy outer structure appears to be breaking apart into powdery clumps.

Volume / size change: There is a clear reduction in height and central mass compared with 15 minutes. The tablet is spreading laterally into separate fragments rather than remaining as one cohesive body.

Dissolution speed and time: Between 15 and 30 minutes, disintegration continues quickly. The tablet shifts from a swollen mass to a fragmented residue with visible side piles.

Physical state change: The structure is now partially collapsed. The tablet no longer behaves as a single solid unit; it is becoming a collection of softened fragments.

Dissolution medium: The medium contains many suspended particles, especially above the tablet. Some downward sediment trails are visible below the support line.

Fragment distribution and density: Fragment density is high near the original tablet site. Larger white clumps appear on the left and right sides, while finer particles are suspended more widely.

## B4 - 45 minutes

Color change: The yellow central region has almost disappeared from the visible tablet body. Most remaining material appears white to off-white.

Shape change: The original tablet form is essentially gone. Only separate low piles of residue remain along the support surface.

Surface texture change: The remaining material is loose, powdery, and clumped. There is no smooth tablet surface left.

Volume / size change: There is a major volume reduction. The central mass has collapsed and dispersed, leaving several small mounds rather than a single tablet residue.

Dissolution speed and time: By 45 minutes, Group B is in a late disintegration stage. Compared with Group A at the same time, Group B has disintegrated much more completely.

Physical state change: The tablet has changed into fragmented, powder-like residue. The cohesive structure has almost completely failed.

Dissolution medium: Fine particles remain suspended throughout the field. Sediment-like material is visible below the support surface.

Fragment distribution and density: Fragments are spread horizontally across the original tablet area. Several small piles remain, with the largest pile on the right side. The density is no longer concentrated in one central mound.

## B5 - 60 minutes

Color change: The remaining visible residue is mostly white/off-white. The earlier yellow center is no longer clearly visible.

Shape change: No intact tablet body remains. Only small, separated residue piles are visible.

Surface texture change: The remaining material is powdery and granular. There is no recognizable tablet surface.

Volume / size change: The visible volume is greatly reduced compared with 0, 15, and 30 minutes. Only a thin line of residue and several small clumps remain.

Dissolution speed and time: By 60 minutes, Group B has nearly completed visible disintegration. Remaining material appears to be residual insoluble or slowly dispersing particles.

Physical state change: The tablet has transitioned from an intact solid to dispersed powder/sediment. The original solid structure has fully collapsed.

Dissolution medium: Fine suspended particles remain visible in the medium, and sediment is present beneath the support region.

Fragment distribution and density: Fragments are low-density and spread along the support surface. The largest residue is no longer central; the material is distributed as small separated piles.

## Overall Group B Disintegration Pattern

Group B begins as a pale yellow, flat cylindrical tablet. It swells quickly by 15 minutes, exposing a yellow internal region while developing a white, fluffy, porous outer layer. By 30 minutes, the tablet has collapsed into an irregular mound with side fragments. By 45 minutes, the original tablet structure is almost gone. By 60 minutes, only small powdery residue piles remain.

The key visual pattern is:

Intact pale-yellow flat tablet.

Swollen white/yellow porous mass.

Collapsed low mound.

Separated powdery residue.

Nearly complete visible disintegration.

## Initial Comparison: Group A vs Group B

Initial appearance at 0 minutes: Group A is white, thicker, and biconvex, with rounded upper and lower contours. Group B is pale yellow, wider, flatter, and more cylindrical, with a straighter side wall. This is a major difference because the tablets already look different before disintegration has significantly progressed.

Size and shape evolution: Group A forms a rounded dome-like mound that remains relatively cohesive through 60 minutes. Group B forms a swollen mass at 15 minutes but then collapses much more strongly; by 45 to 60 minutes it is mostly reduced to small separated residue piles.

Color behavior: Group A starts white, then exposes a pale yellow interior after 15 minutes. Yellow remains visible in the central residue even at 60 minutes. Group B starts pale yellow already. During disintegration, a white swollen layer appears around a yellow interior, but by 45 to 60 minutes the visible yellow largely disappears.

Surface texture and fragmentation: Group A becomes fluffy and granular, but the residue remains as one main cohesive mound for the whole 60-minute sequence. Group B becomes fluffy and granular at 15 to 30 minutes, then breaks down into multiple separated clumps by 45 to 60 minutes.

Dissolution speed: Group A changes quickly from 0 to 15 minutes, but after that the remaining mass erodes slowly. Group B changes quickly from 0 to 30 minutes, and by 45 to 60 minutes most of the tablet body has disappeared.

Based on the visual evidence, Group A and Group B are unlikely to be images of the same tablet formulation. They may possibly contain the same labeled active ingredient, but visually they behave like different formulations, different manufacturers, different excipient systems, or possibly different drugs. The strongest evidence is the difference in initial color, initial shape, persistence of Group A's residue at 60 minutes, and the more complete collapse of Group B by 60 minutes.

# 4. Final Eight-Dimension Comparison and Judgment

User

Please compare Groups A and B based on their disintegration characteristics across the eight dimensions to determine whether they are from the same drug or not, and explain why you think so.

Assistant

## Final Conclusion

Based on the visible disintegration behavior across the eight dimensions, Groups A and B are unlikely to be the same tablet product/formulation.

They could theoretically share the same labeled active ingredient, but from the images alone I would judge them as different formulations, different manufacturers, different strengths, or possibly different drugs. The main reason is that their initial appearance, shape, color sequence, erosion pattern, and final residue at 60 minutes are clearly different.

## Eight-Dimension Comparison

| Dimension | Group A | Group B | Interpretation |
| --- | --- | --- | --- |
| 1. Color change | Starts white at 0 minutes. Yellow material becomes exposed after disintegration begins and remains visible through 60 minutes. | Starts pale yellow at 0 minutes. At 15-30 minutes, white swollen material surrounds a yellow center, but by 45-60 minutes the visible residue is mostly white/off-white. | The color sequence is different. Group A changes from white to yellow-exposed residue, while Group B begins yellow and later loses the visible yellow region. |
| 2. Shape change | Starts as a thick, biconvex tablet. After swelling, it becomes a rounded dome-like mass and remains one main body through 60 minutes. | Starts as a wide, flatter cylindrical tablet. It collapses into a low mound by 30 minutes and separates into small residue piles by 45-60 minutes. | Initial shape and later shape evolution do not match. This is strong evidence against the same formulation. |
| 3. Surface texture change | Smooth at 0 minutes, then rough, porous, fluffy, and granular. The surface remains part of a cohesive mound. | Slightly matte at 0 minutes, then rapidly fluffy and powdery. By 45-60 minutes, no continuous tablet surface remains. | Both become porous and powdery, but Group B loses surface integrity much faster. |
| 4. Volume / size change | Large visible residue remains even at 60 minutes. The tablet loses original shape but retains substantial mass. | Major volume loss by 45 minutes. At 60 minutes, only small separated piles remain. | Group B disintegrates and disperses much more completely. |
| 5. Dissolution speed and time | Rapid change from 0 to 15 minutes, then slower erosion from 15 to 60 minutes. | Rapid change from 0 to 15 minutes and continued strong collapse from 15 to 45 minutes. Nearly complete visible breakdown by 60 minutes. | Group B has a faster and more complete visible disintegration profile. |
| 6. Physical state change | Compact solid to swollen porous mass to persistent soft cohesive residue. | Compact solid to swollen soft mass to collapsed fragments to scattered powder/sediment. | Group A retains cohesion; Group B loses cohesion and fragments more completely. |
| 7. Dissolution medium | Suspended fine particles and downward sediment trails are visible, but the main residue remains localized. | Suspended particles are also visible, but more material disperses from the tablet body as the structure collapses. | Both release particles, but Group B produces more dispersed residue by late time points. |
| 8. Fragment distribution and density | Fragments remain concentrated around the original tablet site. One main mound persists, with sediment below. | Fragments spread horizontally into multiple small piles. By 60 minutes, density is low and distributed along the support surface. | Fragment distribution is different: Group A is centralized, while Group B is scattered and depleted. |

## Why They Are Judged as Different

The strongest evidence comes from the 0-minute images. Before disintegration has significantly progressed, the tablets already look different:

Group A is white, thicker, biconvex, and compact.

Group B is pale yellow, flatter, wider, and more cylindrical.

These differences are not caused by dissolution because they are visible at the starting point.

The second strongest evidence is the 60-minute endpoint:

Group A still has a large, dome-like, pale yellow/off-white porous mass.

Group B has mostly collapsed into small separated white residue piles, with no intact central body.

The full time-course pattern is also different:

Group A: intact white tablet to yellow-exposed swollen mound to persistent cohesive residue.

Group B: pale yellow flat tablet to white/yellow swollen mass to rapid collapse to scattered powdery residue.

## Judgment

Groups A and B are not visually consistent with the same tablet formulation. The best conclusion from the images is that they are likely different tablet products/formulations rather than the same drug tablet.

However, image analysis cannot chemically confirm whether the active ingredient is the same. To confirm whether both are truly Doxycycline Hyclate, chemical testing such as HPLC, UV assay, or official dissolution testing would be needed.